SIMUALTION 3

CODE :

clc;

clear;

close all;

%% =========================================================

% DESIGN OF IIR FILTER USING

% 1. IMPULSE INVARIANCE

% 2. BILINEAR TRANSFORMATION

% ==========================================================

%% Given Specifications

Fs = 1000;              % Sampling frequency (Hz)

T = 1/Fs;               % Sampling period

Wp = 200;               % Analog passband edge (rad/s)

Ws = 300;               % Analog stopband edge (rad/s)

Ap = 1;                 % Passband attenuation (dB)

As = 40;                % Stopband attenuation (dB)

fprintf('Sampling Frequency = %d Hz\n', Fs);

fprintf('Analog Passband Edge = %d rad/s\n', Wp);

fprintf('Analog Stopband Edge = %d rad/s\n', Ws);

fprintf('Passband Attenuation = %d dB\n', Ap);

fprintf('Stopband Attenuation = %d dB\n\n', As);

%% =========================================================

% PART 1: ANALOG BUTTERWORTH FILTER

% ==========================================================

% Find minimum order of analog Butterworth filter

[N, Wn] = buttord(Wp, Ws, Ap, As, 's');

% Design analog Butterworth filter

[ba, aa] = butter(N, Wn, 's');

fprintf('Analog Butterworth Filter Order = %d\n', N);

fprintf('Analog cutoff frequency = %.4f rad/s\n\n', Wn);

disp('Analog numerator coefficients ba = ');

disp(ba);

disp('Analog denominator coefficients aa = ');

disp(aa);

%% =========================================================

% PART 2: IMPULSE INVARIANCE

% ==========================================================

% Convert analog filter to digital using impulse invariance

[bII, aII] = impinvar(ba, aa, Fs);

fprintf('\n============================================\n');

fprintf('IMPULSE INVARIANCE\n');

fprintf('============================================\n');

disp('Digital numerator coefficients bII = ');

disp(bII);

disp('Digital denominator coefficients aII = ');

disp(aII);

%% =========================================================

% PART 3: BILINEAR TRANSFORMATION

% WITH PREWARPING

% ==========================================================

% Desired digital frequencies corresponding to analog edges

fp = (Fs/pi) * atan(Wp/(2*Fs));

fs_stop = (Fs/pi) * atan(Ws/(2*Fs));

fprintf('\nDesired digital passband edge = %.4f Hz\n', fp);

fprintf('Desired digital stopband edge = %.4f Hz\n\n', fs_stop);

% Prewarped analog frequencies

Wp_pre = 2*Fs*tan(pi*fp/Fs);

Ws_pre = 2*Fs*tan(pi*fs_stop/Fs);

fprintf('Prewarped passband frequency = %.4f rad/s\n', Wp_pre);

fprintf('Prewarped stopband frequency = %.4f rad/s\n', Ws_pre);

% Find filter order using prewarped frequencies

[N_BL, Wn_BL] = buttord(Wp_pre, Ws_pre, Ap, As, 's');

% Design analog Butterworth filter for bilinear transformation

[ba_BL, aa_BL] = butter(N_BL, Wn_BL, 's');

% Apply bilinear transformation

[bBL, aBL] = bilinear(ba_BL, aa_BL, Fs);

fprintf('\n============================================\n');

fprintf('BILINEAR TRANSFORMATION\n');

fprintf('============================================\n');

fprintf('Bilinear Filter Order = %d\n', N_BL);

disp('Digital numerator coefficients bBL = ');

disp(bBL);

disp('Digital denominator coefficients aBL = ');

disp(aBL);

%% =========================================================

% PART 4: FREQUENCY RESPONSE

% ==========================================================

% Frequency response of Impulse Invariance filter

[HII, f] = freqz(bII, aII, 2048, Fs);

% Frequency response of Bilinear filter

[HBL, ~] = freqz(bBL, aBL, 2048, Fs);

%% =========================================================

% PART 5: MAGNITUDE RESPONSE

% ==========================================================

figure;

plot(f, 20*log10(abs(HII)), 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(HBL)), '--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response Comparison');

legend('Impulse Invariance', ...

'Bilinear Transformation', ...

'Location','best');

xlim([0 150]);

%% =========================================================

% PART 6: PHASE RESPONSE

% ==========================================================

figure;

plot(f, unwrap(angle(HII)), 'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(HBL)), '--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response Comparison');

legend('Impulse Invariance', ...

'Bilinear Transformation', ...

'Location','best');

xlim([0 150]);

%% =========================================================

% PART 7: MARK PASSBAND AND STOPBAND EDGES

% ==========================================================

figure;

plot(f, 20*log10(abs(HII)), 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(HBL)), '--', 'LineWidth', 1.5);

xline(fp, ':', 'Passband Edge');

xline(fs_stop, ':', 'Stopband Edge');

yline(-Ap, ':', 'Ap = 1 dB');

yline(-As, ':', 'As = 40 dB');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('IIR Filter Response with Design Specifications');

legend('Impulse Invariance', ...

'Bilinear Transformation', ...

'Location','best');

xlim([0 150]);

%% =========================================================

% PART 8: DISPLAY COMPARISON

% ==========================================================

fprintf('\n============================================\n');

fprintf('FILTER COMPARISON\n');

fprintf('============================================\n');

fprintf('Impulse Invariance Order = %d\n', N);

fprintf('Bilinear Transformation Order = %d\n', N_BL);

fprintf('\nImpulse Invariance coefficients:\n');

disp(bII);

disp(aII);

fprintf('Bilinear Transformation coefficients:\n');

disp(bBL);

disp(aBL);

fprintf('\n============================================\n');

fprintf('FREQUENCY WARPING OBSERVATION\n');

fprintf('============================================\n');

fprintf(['Bilinear transformation introduces frequency warping.\n' ...

'Prewarping is used to compensate for this effect at\n' ...

'the specified passband and stopband frequencies.\n']);

fprintf('\nExperiment completed successfully.\n');

OUTPUT :