SIMULATION 1:

CODE :

clc;

clear;

close all;

%% Given Specifications

Fs = 1000;       % Sampling frequency in Hz

Ap = 1;           % Passband attenuation in dB

Ast = 40;         % Stopband attenuation in dB

%% =========================================================

%              BUTTERWORTH LOW-PASS FILTER

% ==========================================================

Fp_LP  = 100;     % Low-pass passband edge (Hz)

Fst_LP = 150;     % Low-pass stopband edge (Hz)

% Normalize frequencies with respect to Nyquist frequency

Wp_LP = Fp_LP/(Fs/2);

Ws_LP = Fst_LP/(Fs/2);

% Calculate minimum filter order and cutoff frequency

[N_LP, Wn_LP] = buttord(Wp_LP, Ws_LP, Ap, Ast);

% Design Butterworth low-pass filter

[b_LP, a_LP] = butter(N_LP, Wn_LP, 'low');

% Display results

fprintf('\n===== BUTTERWORTH LOW-PASS FILTER =====\n');

fprintf('Filter Order = %d\n', N_LP);

fprintf('Normalized Cutoff Frequency = %.4f\n', Wn_LP);

disp('Numerator coefficients (b):');

disp(b_LP);

disp('Denominator coefficients (a):');

disp(a_LP);

% Frequency response

[H_LP, f_LP] = freqz(b_LP, a_LP, 2048, Fs);

figure;

subplot(2,1,1);

plot(f_LP, 20*log10(abs(H_LP)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth IIR Low-Pass Filter - Magnitude Response');

xline(Fp_LP, '--r', 'Passband Edge');

xline(Fst_LP, '--g', 'Stopband Edge');

subplot(2,1,2);

plot(f_LP, unwrap(angle(H_LP))*180/pi);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Butterworth IIR Low-Pass Filter - Phase Response');

%% =========================================================

%              BUTTERWORTH HIGH-PASS FILTER

% ==========================================================

Fst_HP = 200;     % High-pass stopband edge (Hz)

Fp_HP  = 250;     % High-pass passband edge (Hz)

% Normalize frequencies

Wp_HP = Fp_HP/(Fs/2);

Ws_HP = Fst_HP/(Fs/2);

% Calculate minimum filter order and cutoff frequency

[N_HP, Wn_HP] = buttord(Wp_HP, Ws_HP, Ap, Ast);

% Design Butterworth high-pass filter

[b_HP, a_HP] = butter(N_HP, Wn_HP, 'high');

% Display results

fprintf('\n===== BUTTERWORTH HIGH-PASS FILTER =====\n');

fprintf('Filter Order = %d\n', N_HP);

fprintf('Normalized Cutoff Frequency = %.4f\n', Wn_HP);

disp('Numerator coefficients (b):');

disp(b_HP);

disp('Denominator coefficients (a):');

disp(a_HP);

% Frequency response

[H_HP, f_HP] = freqz(b_HP, a_HP, 2048, Fs);

figure;

subplot(2,1,1);

plot(f_HP, 20*log10(abs(H_HP)));

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth IIR High-Pass Filter - Magnitude Response');

xline(Fst_HP, '--r', 'Stopband Edge');

xline(Fp_HP, '--g', 'Passband Edge');

subplot(2,1,2);

plot(f_HP, unwrap(angle(H_HP))*180/pi);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Butterworth IIR High-Pass Filter - Phase Response');

%% =========================================================

%                  COMBINED MAGNITUDE RESPONSE

% ==========================================================

figure;

plot(f_LP, 20*log10(abs(H_LP)), 'LineWidth', 1.5);

hold on;

plot(f_HP, 20*log10(abs(H_HP)), 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth IIR LPF and HPF Magnitude Responses');

legend('Low-Pass Filter', 'High-Pass Filter');

xlim([0 Fs/2]);

OUTPUT :