SIMULATION 2:

CODE :

clc;

clear;

close all;

%% Given Specifications

Fs = 1000;      % Sampling frequency (Hz)

Fp = 100;       % Passband edge (Hz)

Fst = 150;      % Stopband edge (Hz)

Rp = 1;         % Passband ripple (dB)

Rs = 40;        % Stopband attenuation (dB)

%% Normalize frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

%% =========================================================

%       CHEBYSHEV TYPE-I LOW-PASS FILTER

% ==========================================================

% Find minimum filter order and cutoff frequency

[N_LP, Wn_LP] = cheb1ord(Wp, Ws, Rp, Rs);

% Design Chebyshev Type-I Low-Pass Filter

[b_LP, a_LP] = cheby1(N_LP, Rp, Wn_LP, 'low');

% Display results

fprintf('\n========================================\n');

fprintf('CHEBYSHEV TYPE-I LOW-PASS FILTER\n');

fprintf('========================================\n');

fprintf('Filter Order = %d\n', N_LP);

fprintf('Normalized Cutoff Frequency = %.4f\n', Wn_LP);

disp('Numerator coefficients (b):');

disp(b_LP);

disp('Denominator coefficients (a):');

disp(a_LP);

% Frequency response

[H_LP, f_LP] = freqz(b_LP, a_LP, 2048, Fs);

% Plot LPF response

figure;

subplot(2,1,1);

plot(f_LP, 20*log10(abs(H_LP)), 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Chebyshev Type-I Low-Pass Filter - Magnitude Response');

xline(Fp, '--r', 'Passband Edge');

xline(Fst, '--g', 'Stopband Edge');

subplot(2,1,2);

plot(f_LP, unwrap(angle(H_LP))*180/pi, 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Chebyshev Type-I Low-Pass Filter - Phase Response');

%% =========================================================

%       CHEBYSHEV TYPE-I HIGH-PASS FILTER

% ==========================================================

% For High-Pass Filter:

% Stopband edge = 100 Hz

% Passband edge = 150 Hz

Fst_HP = 100;

Fp_HP = 150;

% Normalize frequencies

Ws_HP = Fst_HP/(Fs/2);

Wp_HP = Fp_HP/(Fs/2);

% Find minimum filter order

[N_HP, Wn_HP] = cheb1ord(Wp_HP, Ws_HP, Rp, Rs);

% Design Chebyshev Type-I High-Pass Filter

[b_HP, a_HP] = cheby1(N_HP, Rp, Wn_HP, 'high');

% Display results

fprintf('\n========================================\n');

fprintf('CHEBYSHEV TYPE-I HIGH-PASS FILTER\n');

fprintf('========================================\n');

fprintf('Filter Order = %d\n', N_HP);

fprintf('Normalized Cutoff Frequency = %.4f\n', Wn_HP);

disp('Numerator coefficients (b):');

disp(b_HP);

disp('Denominator coefficients (a):');

disp(a_HP);

% Frequency response

[H_HP, f_HP] = freqz(b_HP, a_HP, 2048, Fs);

% Plot HPF response

figure;

subplot(2,1,1);

plot(f_HP, 20*log10(abs(H_HP)), 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Chebyshev Type-I High-Pass Filter - Magnitude Response');

xline(Fst_HP, '--r', 'Stopband Edge');

xline(Fp_HP, '--g', 'Passband Edge');

subplot(2,1,2);

plot(f_HP, unwrap(angle(H_HP))*180/pi, 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Chebyshev Type-I High-Pass Filter - Phase Response');

%% =========================================================

%       BUTTERWORTH COMPARISON - LOW PASS

% ==========================================================

[N_BW_LP, Wn_BW_LP] = buttord(Wp, Ws, Rp, Rs);

[b_BW_LP, a_BW_LP] = butter(N_BW_LP, Wn_BW_LP, 'low');

[H_BW_LP, f_BW_LP] = freqz(b_BW_LP, a_BW_LP, 2048, Fs);

fprintf('\n========================================\n');

fprintf('BUTTERWORTH LOW-PASS COMPARISON\n');

fprintf('========================================\n');

fprintf('Butterworth Filter Order = %d\n', N_BW_LP);

%% Plot Chebyshev vs Butterworth LPF

figure;

plot(f_LP, 20*log10(abs(H_LP)), 'LineWidth', 1.5);

hold on;

plot(f_BW_LP, 20*log10(abs(H_BW_LP)), 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Chebyshev Type-I vs Butterworth Low-Pass Filter');

legend('Chebyshev Type-I', 'Butterworth');

xline(Fp, '--r', 'Passband Edge');

xline(Fst, '--g', 'Stopband Edge');

xlim([0 Fs/2]);

%% =========================================================

%       DISPLAY COMPARISON OF FILTER ORDERS

% ==========================================================

fprintf('\n========================================\n');

fprintf('FILTER ORDER COMPARISON\n');

fprintf('========================================\n');

fprintf('Chebyshev Type-I LPF Order = %d\n', N_LP);

fprintf('Butterworth LPF Order      = %d\n', N_BW_LP);

fprintf('Chebyshev Type-I HPF Order = %d\n', N_HP);

OUTPUT: