SIMULATION 4

CODE :

clc;

clear;

close all;

%% =========================================================

% SIMULATION EXPERIMENT - 4

% Comparative Analysis of Butterworth, Chebyshev-I

% and Elliptic IIR Filters

% ==========================================================

%% Given Specifications

Fs  = 1000;     % Sampling frequency in Hz

Fp  = 100;      % Passband edge frequency in Hz

Fst = 150;      % Stopband edge frequency in Hz

Rp  = 1;        % Passband ripple in dB

Rs  = 40;       % Stopband attenuation in dB

fprintf('============================================\n');

fprintf('IIR FILTER DESIGN SPECIFICATIONS\n');

fprintf('============================================\n');

fprintf('Sampling Frequency     = %d Hz\n', Fs);

fprintf('Passband Edge          = %d Hz\n', Fp);

fprintf('Stopband Edge          = %d Hz\n', Fst);

fprintf('Passband Ripple        = %d dB\n', Rp);

fprintf('Stopband Attenuation   = %d dB\n', Rs);

%% Normalized frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

fprintf('\nNormalized Passband Frequency = %.3f\n', Wp);

fprintf('Normalized Stopband Frequency = %.3f\n', Ws);

%% Transition Bandwidth

DeltaF = Fst - Fp;

fprintf('\nTransition Bandwidth = %d Hz\n', DeltaF);

%% =========================================================

% 1. BUTTERWORTH FILTER

% ==========================================================

[N_B, Wn_B] = buttord(Wp, Ws, Rp, Rs);

[b_B, a_B] = butter(N_B, Wn_B, 'low');

fprintf('\n============================================\n');

fprintf('BUTTERWORTH FILTER\n');

fprintf('============================================\n');

fprintf('Filter Order = %d\n', N_B);

fprintf('Cutoff Frequency = %.4f Hz\n', Wn_B*(Fs/2));

disp('Numerator coefficients b_B = ');

disp(b_B);

disp('Denominator coefficients a_B = ');

disp(a_B);

%% =========================================================

% 2. CHEBYSHEV TYPE-I FILTER

% ==========================================================

[N_C, Wn_C] = cheb1ord(Wp, Ws, Rp, Rs);

[b_C, a_C] = cheby1(N_C, Rp, Wn_C, 'low');

fprintf('\n============================================\n');

fprintf('CHEBYSHEV TYPE-I FILTER\n');

fprintf('============================================\n');

fprintf('Filter Order = %d\n', N_C);

fprintf('Cutoff Frequency = %.4f Hz\n', Wn_C*(Fs/2));

disp('Numerator coefficients b_C = ');

disp(b_C);

disp('Denominator coefficients a_C = ');

disp(a_C);

%% =========================================================

% 3. ELLIPTIC FILTER

% ==========================================================

[N_E, Wn_E] = ellipord(Wp, Ws, Rp, Rs);

[b_E, a_E] = ellip(N_E, Rp, Rs, Wn_E, 'low');

fprintf('\n============================================\n');

fprintf('ELLIPTIC FILTER\n');

fprintf('============================================\n');

fprintf('Filter Order = %d\n', N_E);

fprintf('Cutoff Frequency = %.4f Hz\n', Wn_E*(Fs/2));

disp('Numerator coefficients b_E = ');

disp(b_E);

disp('Denominator coefficients a_E = ');

disp(a_E);

%% =========================================================

% 4. FREQUENCY RESPONSES

% ==========================================================

[H_B, f] = freqz(b_B, a_B, 2048, Fs);

[H_C, ~] = freqz(b_C, a_C, 2048, Fs);

[H_E, ~] = freqz(b_E, a_E, 2048, Fs);

%% =========================================================

% 5. MAGNITUDE RESPONSE

% ==========================================================

figure;

plot(f, 20*log10(abs(H_B)), 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(H_C)), '--', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H_E)), ':', 'LineWidth', 1.8);

xline(Fp, '--');

xline(Fst, '--');

yline(-Rp, ':');

yline(-Rs, ':');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Comparison of IIR Filter Magnitude Responses');

legend('Butterworth', ...

'Chebyshev Type-I', ...

'Elliptic', ...

'Passband Edge', ...

'Stopband Edge', ...

'Location','best');

xlim([0 250]);

ylim([-80 5]);

%% =========================================================

% 6. PHASE RESPONSE

% ==========================================================

figure;

plot(f, unwrap(angle(H_B)), 'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(H_C)), '--', 'LineWidth', 1.5);

plot(f, unwrap(angle(H_E)), ':', 'LineWidth', 1.8);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response Comparison');

legend('Butterworth', ...

'Chebyshev Type-I', ...

'Elliptic', ...

'Location','best');

xlim([0 250]);

%% =========================================================

% 7. COMPARATIVE TABLE

% ==========================================================

Filter_Name = {'Butterworth';

'Chebyshev Type-I';

'Elliptic'};

Filter_Order = [N_B;

N_C;

N_E];

Passband_Ripple_dB = [Rp;

Rp;

Rp];

Stopband_Attenuation_dB = [Rs;

Rs;

Rs];

Transition_Bandwidth_Hz = [DeltaF;

DeltaF;

DeltaF];

Comparison_Table = table(Filter_Name, ...

Filter_Order, ...

Passband_Ripple_dB, ...

Stopband_Attenuation_dB, ...

Transition_Bandwidth_Hz);

fprintf('\n============================================\n');

fprintf('COMPARATIVE TABLE\n');

fprintf('============================================\n');

disp(Comparison_Table);

%% =========================================================

% 8. FIND ACTUAL RESPONSE VALUES

% ==========================================================

% Response at passband edge

[~, idx_fp] = min(abs(f-Fp));

Mag_B_Fp = 20*log10(abs(H_B(idx_fp)));

Mag_C_Fp = 20*log10(abs(H_C(idx_fp)));

Mag_E_Fp = 20*log10(abs(H_E(idx_fp)));

% Response at stopband edge

[~, idx_fst] = min(abs(f-Fst));

Mag_B_Fst = 20*log10(abs(H_B(idx_fst)));

Mag_C_Fst = 20*log10(abs(H_C(idx_fst)));

Mag_E_Fst = 20*log10(abs(H_E(idx_fst)));

fprintf('\n============================================\n');

fprintf('RESPONSE AT SPECIFIED EDGES\n');

fprintf('============================================\n');

fprintf('\nAt Passband Edge = %d Hz\n', Fp);

fprintf('Butterworth      = %.2f dB\n', Mag_B_Fp);

fprintf('Chebyshev Type-I = %.2f dB\n', Mag_C_Fp);

fprintf('Elliptic         = %.2f dB\n', Mag_E_Fp);

fprintf('\nAt Stopband Edge = %d Hz\n', Fst);

fprintf('Butterworth      = %.2f dB\n', Mag_B_Fst);

fprintf('Chebyshev Type-I = %.2f dB\n', Mag_C_Fst);

fprintf('Elliptic         = %.2f dB\n', Mag_E_Fst);

%% =========================================================

% 9. FINAL COMPARISON

% ==========================================================

fprintf('\n============================================\n');

fprintf('FINAL COMPARISON\n');

fprintf('============================================\n');

fprintf('Butterworth Order      = %d\n', N_B);

fprintf('Chebyshev Type-I Order = %d\n', N_C);

fprintf('Elliptic Order         = %d\n', N_E);

fprintf('\nTransition Bandwidth = %d Hz\n', DeltaF);

fprintf('\nObservation:\n');

fprintf(['Butterworth provides a smooth response without ripple.\n' ...

'Chebyshev Type-I provides a sharper transition but has\n' ...

'passband ripple.\n' ...

'Elliptic provides the sharpest transition with ripple\n' ...

'in both passband and stopband.\n']);

fprintf('\nExperiment completed successfully.\n');

OUTPUT :