SIMULATION 5

CODE :

clc;

clear;

close all;

%% =========================================================

% EXPERIMENT:

% REALIZATION OF IIR FILTERS USING

% DIRECT FORM-I, DIRECT FORM-II,

% CASCADE AND PARALLEL STRUCTURES

% ==========================================================

%% Given Specifications

Fs = 1000;      % Sampling frequency in Hz

Fp = 100;       % Passband edge in Hz

Fst = 150;      % Stopband edge in Hz

Rp = 1;         % Passband ripple in dB

Rs = 40;        % Stopband attenuation in dB

%% Normalized frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

%% =========================================================

% DESIGN BUTTERWORTH IIR FILTER

% (Coefficients obtained from Experiment 1)

% ==========================================================

[N, Wn] = buttord(Wp, Ws, Rp, Rs);

[b, a] = butter(N, Wn, 'low');

fprintf('============================================\n');

fprintf('BUTTERWORTH IIR FILTER\n');

fprintf('============================================\n');

fprintf('Filter Order = %d\n', N);

fprintf('Sampling Frequency = %d Hz\n', Fs);

disp('Numerator coefficients b = ');

disp(b);

disp('Denominator coefficients a = ');

disp(a);

%% =========================================================

% DIRECT FORM-I

% ==========================================================

% Direct Form-I difference equation:

%

% y(n) = b(1)x(n) + b(2)x(n-1) + ...

%        - a(2)y(n-1) - a(3)y(n-2) ...

Nsample = 50;

% Unit impulse input

x = [1 zeros(1,Nsample-1)];

% Direct Form-I output

y_DF1 = zeros(size(x));

for n = 1:Nsample

% Feed-forward part

for k = 1:length(b)

if (n-k+1) > 0

y_DF1(n) = y_DF1(n) + b(k)*x(n-k+1);

end

end

% Feedback part

for k = 2:length(a)

if (n-k+1) > 0

y_DF1(n) = y_DF1(n) - a(k)*y_DF1(n-k+1);

end

end

end

fprintf('\n============================================\n');

fprintf('DIRECT FORM-I\n');

fprintf('============================================\n');

disp('Numerator coefficients b = ');

disp(b);

disp('Denominator coefficients a = ');

disp(a);

%% =========================================================

% DIRECT FORM-II

% ==========================================================

% Direct Form-II uses a single delay line.

y_DF2 = zeros(size(x));

% State vector

w = zeros(1, max(length(a),length(b))-1);

for n = 1:Nsample

% Current internal state

w0 = x(n);

% Feedback

for k = 2:length(a)

if (k-1) <= length(w)

w0 = w0 - a(k)*w(k-1);

end

end

% Output

y_DF2(n) = b(1)*w0;

for k = 2:length(b)

if (k-1) <= length(w)

y_DF2(n) = y_DF2(n) + b(k)*w(k-1);

end

end

% Shift delay elements

if length(w) > 1

w(2:end) = w(1:end-1);

end

if length(w) >= 1

w(1) = w0;

end

end

fprintf('\n============================================\n');

fprintf('DIRECT FORM-II\n');

fprintf('============================================\n');

disp('Direct Form-II uses the same b and a coefficients.');

disp('It uses a single delay line (canonical structure).');

%% =========================================================

% CASCADE REALIZATION

% SECOND-ORDER SECTIONS

% ==========================================================

[sos, g] = tf2sos(b,a);

fprintf('\n============================================\n');

fprintf('CASCADE REALIZATION\n');

fprintf('============================================\n');

disp('Second Order Sections (SOS) = ');

disp(sos);

fprintf('Overall gain = %.10f\n',g);

% Cascade output

y_Cascade = sosfilt(sos,x')';

% Apply overall gain

y_Cascade = g*y_Cascade;

%% =========================================================

% PARALLEL REALIZATION

% PARTIAL FRACTION DECOMPOSITION

% ==========================================================

[r,p,k] = residuez(b,a);

fprintf('\n============================================\n');

fprintf('PARALLEL REALIZATION\n');

fprintf('============================================\n');

disp('Residues r = ');

disp(r);

disp('Poles p = ');

disp(p);

disp('Direct term k = ');

disp(k);

%% =========================================================

% PARALLEL OUTPUT

% ==========================================================

% Use MATLAB residue realization

y_Parallel = zeros(size(x));

for m = 1:length(r)

% First order branch:

% r/(1-p*z^-1)

[num_branch, den_branch] = residuez(r(m),p(m),[]);

y_branch = filter(num_branch,den_branch,x);

y_Parallel = y_Parallel + y_branch;

end

% Add direct polynomial part if present

if ~isempty(k)

y_Parallel = y_Parallel + filter(k,1,x);

end

%% =========================================================

% MATLAB REFERENCE OUTPUT

% ==========================================================

y_reference = filter(b,a,x);

%% =========================================================

% PLOT IMPULSE RESPONSES

% ==========================================================

n = 0:Nsample-1;

figure;

stem(n,y_reference,'filled');

hold on;

stem(n,y_DF1,'--');

stem(n,y_DF2,':');

stem(n,y_Cascade,'-.');

stem(n,y_Parallel,'-');

grid on;

xlabel('n');

ylabel('Amplitude');

title('Impulse Response Comparison');

legend('Original IIR', ...

'Direct Form-I', ...

'Direct Form-II', ...

'Cascade', ...

'Parallel');

hold off;

%% =========================================================

% INDIVIDUAL RESPONSE PLOT

% ==========================================================

figure;

subplot(2,2,1);

stem(n,y_DF1,'filled');

grid on;

title('Direct Form-I');

xlabel('n');

ylabel('Amplitude');

subplot(2,2,2);

stem(n,y_DF2,'filled');

grid on;

title('Direct Form-II');

xlabel('n');

ylabel('Amplitude');

subplot(2,2,3);

stem(n,y_Cascade,'filled');

grid on;

title('Cascade Realization');

xlabel('n');

ylabel('Amplitude');

subplot(2,2,4);

stem(n,y_Parallel,'filled');

grid on;

title('Parallel Realization');

xlabel('n');

ylabel('Amplitude');

%% =========================================================

% ERROR CALCULATION

% ==========================================================

error_DF1 = max(abs(y_reference-y_DF1));

error_DF2 = max(abs(y_reference-y_DF2));

error_Cascade = max(abs(y_reference-y_Cascade));

error_Parallel = max(abs(y_reference-y_Parallel));

fprintf('\n============================================\n');

fprintf('NUMERICAL ERROR COMPARISON\n');

fprintf('============================================\n');

fprintf('Direct Form-I Maximum Error  = %.10e\n',error_DF1);

fprintf('Direct Form-II Maximum Error = %.10e\n',error_DF2);

fprintf('Cascade Maximum Error         = %.10e\n',error_Cascade);

fprintf('Parallel Maximum Error        = %.10e\n',error_Parallel);

%% =========================================================

% FREQUENCY RESPONSE

% ==========================================================

[H,f] = freqz(b,a,1024,Fs);

figure;

plot(f,20*log10(abs(H)),'LineWidth',1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Frequency Response of Realized IIR Filter');

xlim([0 250]);

%% =========================================================

% FINAL CONCLUSION

% ==========================================================

fprintf('\n============================================\n');

fprintf('CONCLUSION\n');

fprintf('============================================\n');

fprintf(['The Direct Form-I, Direct Form-II, Cascade and\n' ...

'Parallel realizations produce equivalent IIR filter\n' ...

'responses within numerical precision.\n']);

fprintf(['Direct Form-I uses separate feed-forward and feedback\n' ...

'delay lines, while Direct Form-II uses a single delay line.\n']);

fprintf(['Cascade realization represents the filter using\n' ...

'second-order sections, while Parallel realization\n' ...

'uses partial-fraction branches.\n']);

fprintf('\nExperiment completed successfully.\n');

OUTPUT :